1.Linear scales:

1.1. Heidenhain: Iskra

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS303 LS303C |  |  | +/-10 um | 20um | Depending of interpolation electronics | 70-1240mm 70-2040mm: with mounting bar | sine current: 11uA | LS303: 2x35mm,Lm<1120 2x45mm,Lm>1120 LS303C: Distance coded RI | TGM130/SI with LS303 TGM131/SI with LS303C |  | TGM130/131/SI: +has: +/- 5um, and +/-3um accuracy |
| LS323 LS323C |  |  | +/-10 um | 20um | 10um, 5um | 70-1240mm | Square wave: 5V TTL | LS323: every 50mm via magnet selector LS323C: Distance coded RI | TGM130/DS with LS323 TGM131/DS with LS323C |  | TGM130/131/DS: +has. +/-5um and +/-3um accuracy, +has. 2um, 1um, 0.5 um resolution |
| LS603 LS603C |  |  | +/-10 um | 20um | Depending of interpolation electronics | 170-3040mm | sine current: 11uA | LS603: each 50mm LS603C: coded RI | ## TGM170/SI with LS603 and LS603C TGM170 width: 37 mm Mounting element C !!! |  | TGM170/SI: + has: accuracy +/-5um, 3um |
| LS623 LS623C |  |  | +/-10 um | 20um | 10um, 5um | 170-3040mm | Square wave: 5V TTL | LS623: each 50mm;or by request on distance 50mm LS623C: coded RI | # TGM170/DS with LS623 and LS623C Mounting tolerance is 0.3 LS623 2 mm Mounting element C !!! |  | TGM170/DS: +has: accuracy +/- 5um, +/-3um +has: resolutions 1um, 0.5um |
| LS704 LS704C |  |  | +/-5um, +/-3um up to Lm=1240mm | 20um | Depending of interpolation electronics | 170-3040mm | LS704,LS704C: sine current 7-16 uA | LS704: every 50mm (selection) via magnet selector LS704C: coded RI | # TGM170/SI Mounting element A !!! |  | TGM170/SI: -no RI via magnet selector -no thermal compensation |

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS406 LS406C |  |  | +/-5um, +/-3um | 20um | Depending of interpolation electronics | 70-1240mm 70-2040mm: with mounting bar | LS406,LS406C: sine current:11uA | LS406: every 50mm via magnetic selector LS406C: Distance coded RI | TGM133/SI with LS406 and LS406C |  | TGM133/SI: -Lm+138 also with mounting bar -no RI via magnet selector -no thermal comp. Bar |
| LS486 LS486C |  |  | +/-5um, +/-3um | 20um | Depending of interpolation electronics | 70-1240mm 70-2040mm: with mounting bar | LS486,LS486C: sine: 1Vpp | LS486: every 50mm via magnetic selector LS486C: Distance coded RI | TGM133/SV with LS486 and LS486C |  | TGM133/SV: -Lm+138 also with mounting bar -no RI via magnet selector |
| LS476 LS476C |  |  | +/-5um, +/-3um | 20um | 1um, 0.5um | 70-1240mm 70-2040mm: with mounting bar | square wave: 5VTTL | LS476: every 50mm via magnetic selector LS476C: Distance coded RI | TGM133/DS with LS476 and LS476C |  | TGM133/DS: -Lm+138 also with mounting bar |
| LS106 LS106C |  |  | +/-5um, +/-3um | 20um | Depending of interpolation electronics | 140-3040mm | LS106, LS106C: sine current 11uA | LS106: every 50mm via magnet selector LS106C: coded RI | TGM173/SI with LS106/LS106C |  | TGM173/SI -no RI via magnet selector |
| LS176 LS176C |  |  | +/-5um, +/-3um | 20um | 1um, 0.5um | 140-3040mm | LS176,LS176C: square wave: 5V TTL | LS176: every 50mm via magnet selector LS176C: coded RI | TGM173/DS with LS176/LS176C |  | TGM173/DS: -no RI via magnet selector |
| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| LS186 LS186C |  |  | +/-5um, +/-3um | 20um | Depending of interpolation electronics | 140-3040mm | LS186,186C: sine 1Vpp | LS186: every 50mm via magnet selector LS186C: coded RI | TGM173/SV with LS176/LS176C |  | TGM173/SV: -no RI via magnet selector |
| LB302 LS302C |  |  | +/-5um | 40um | Depending of interpolation electronics | 440-30040mm | sine current: 11A sine 1Vpp | LB302: every 50mm via magnet selector LB302C: coded RI | TGM190/SI |  | TGM190/SI: Accuracy grade (+/-m/m) +/-10um |
| LB382 LS382C |  |  | +/-5um | 40um | Depending of interpolation electronics | 440-30040mm | sine voltage: 1Vpp | LB382: every 50mm via magnet selector LB382C: coded RI | TGM190/SV |  | TGM190/SV: Accuracy grade (+/-m/m) +/-10um |

1.2: RSF: Iskra

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSA6701 MSA6703 |  |  | MSA6701: +/-10um,+/-5um +/-3um MSA6703: +/-3um, +/-2um | MSA6701: 20um MSA6703: 10um | Depending of interpolation electronics | 70-2240mm | sine current:11uA | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM130/SI, TGM131/DS: for MSA6701 |  | TGM130/SI, TGM131/SI: -no periode 10um -no accuracy grade 2 um |
| MSA6702 MSA6704 MSA6705 MSA6706 MSA6707 MSA6708 MSA6709 |  |  | MSA6702,: +/-10um,+/-5um MSA6704: +/-10um MSA6705,: +/-2um,+/-3um MSA6706,6707: +/-3um,+/-5um MSA6708: +/-5um MSA6709: +/-2um, +/-3um, +/-5um | MSA6702,6706,6707: 20um MSA6704,6708: 40um MSA6705,6709: 10um | MSA6702:5um MSA6704:10um MSA6705:0.1um MSA6706:1um MSA6707: 0.5um MSA6708:2um MSA6709: 0.25um | 70-2240mm | square wave: 5V TTL (RS422) | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM130/DS, TGM131/DS: for MSA6702MSA6706, MSA6707 | TGM133/DS: for MSA6705 | TGM130/DS, TGM131/DS: -no periode 10um and 40um -no resolution 0,25um -no accuracy grade 2 um |
| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| MSA6711 MSA6712 |  |  | MSA6711: +/-5um, +/-3um, +/-2um MSA6712: +/-2um, +/-3um | MSA6711: 20um MSA6712: 10um | Depending of interpolation electronics | 140-2440mm | sine current 11uA | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM130/SI, TGM131/DS: with MSA6711, 6712 | TGM130/SI, TGM131/DS: -no identical dimensions -no periode 10um -no accuracy grade 2 um |
| MSA6716 MSA6717 |  |  | MSA6716: +/-5um, +/-3um, +/-2um MSA6717: +/-2um, +/-3um, +/-5um | MSA6716: 20um MSA6717: 20um | MSA6716:1um MSA6717: 0.5um | 140-2440mm | square wave: 5V TTL (RS422) | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM130/DS, TGM131/DS: with MSA6716, MSA6717 | TGM130/DS ,TGM131/DS: -no identical dimensions -no accuracy grade 2 um |
| MSA6710 MSA6715 |  |  | MSA6710: +/-10um, MSA6715: +/-5um, +/-10um | MSA6710: 40um MSA6715: 20um | MSA6710: 10um MSA6715: 5um | 140-2440mm | square wave: 5V TTL | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM130/DS, TGM131/DS: with MSA6710, MSA6715 | TGM130/DS, TGM131/DS: -no identical dimensions -no periode 40um |

|  | Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MSA6714 |  |  | +/-10um, +/-5um, +/-3um | 20um | Depending of interpolation electronics | 70-2240mm | sine current signals: 11uA | 2X35mm up to 1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM130/SI, TGM131/SI | TGM130/SI,TGM131/SI: -no identical dimensions -no double rubber protection |
| MSA6800-1 MSA6800-2 |  |  |  | MSA6800-2: +/-10um, +/-5um,+/-3um MSA6800-1: +/-2um, +/-3um | MSA6800-2: 20um MSA6800-1: 10um | Depending of interpolation electronics | 70-1240mm | sine: 1Vpp | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM133/SV with coded RI |  | TGM133/SV -no periode 10um -no accuracy grade 2 um |
| MSA6801 MSA6803 |  |  |  | MSA6801: +/-10um, +/-5um,+/-3um MSA6803: +/-2um, +/-3um | MSA6801: 20um MSA6803: 10um | Depending of interpolation electronics | 70-1240mm | sine current signals: 11uA | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM133/DS with coded RI | TGM130/SI with MSA6801 with standard RI | TGM130/SI: -no identical dimension -no periode 10um |
| MSA6802 MSA6806 MSA6807 MSA6808 MSA6809 |  |  |  | MSA6802: +/-10um, +/-5um,+/-3um MSA6806: +/-3um, +/-5um MSA6807: +/-3um, +/-5um MSA6808: +/-5um MSA6809: +/-2um, +/-3um | MSA6802: 20um MSA6806: 20um, MSA6807: 20um MSA6808: 40um MSA6809: 10um | MSA6802: 5um MSA6806: 1um, MSA6807: 0.5um MSA6808: 2um MSA6809: 0,25um | 70-1240mm | square wave 5V TTL (RS422) | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM133/DS with coded RI | TGM130/DS with with standard RI | TGM130/DS, TGM133/DS: -no resolution 2um, 0.25um |

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSA3700-1 MSA3700-2 MSA3701 MSA3703 |  |  | MSA3701,3700-2: +/-10um, +/-5um,+/-3um MSA3703,3700-1: +/-2um, +/-3um | MSA3701, 3700-2: 20um MSA3703, 3700-1: 10um | Depending of interpolation electronics | 170-3040mm | MSA3701, 3703: sine current signals: 11uA MSA3700-1, 3700-2: sine: 1Vpp | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM170/SI/SV with MSA3701, 3700-2 Mounting element C !!! | TGM173/DS with MSA3701, 3700-2 | TGM173/SI: -no identical dimension -no periode 10um |
| MSA3702 MSA3704 MSA3705 MSA3706 MSA3707 MSA3708 MSA3709 |  |  | MSA3702: +/-10um,+/-5um MSA3706, 3707: +/-3um, +/-5um MSA3704: +/-10um MSA3705, 3709: +/-2um, +/-3um MSA3708: +/-5um, | MSA3702, 3706, 3707: 20um MSA3704, 3708: 40um, MSA3705, 3709: 10um | MSA3702–5um MSA3704-10um MSA3705-0.1um MSA3706-1um MSA3707-0.5um MSA3708-2um MSA3709-0.25um | 170-3040mm | square wave 5V TTL (RS422) | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm | TGM170/DS with MSA3702, 3704, 3706, 3707, 3708 with standard RI Mounting element C !!! | TGM173/DS with MSA3705, 3706, 3707 with coded RI | -no thermal compensation TGM173/DS: -no identical dimension TGM170: -no resolution 2um, 0.25um |
| MSA3711 MSA3713 |  |  | MSA3711: +/-3um, +/-5um,+/-10um MSA3713: +/-2um, +/-3um | MSA3711: 20um MSA3713: 10um, | Depending of interpolation electronics | 170-3040mm | sine current signals: 11uA | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM170/SI with MSA3711 | TGM170/SI: -no identical dimension -no periode 10um |
| MSA3712 MSA3714 MSA3715 MSA3716 MSA3717 MSA3718 MSA3719 |  |  | MSA3712: +/-5um,+/-10um MSA3714: +/-10um MSA3715, 3719: +/-2um, +/-3um MSA3716, 3717: +/-3um, +/-5um MSA3718: +/-5um | MSA3712, 3716, 3717: 20um MSA3714, 3718: 40um MSA3715, 3719: 10um | MSA3712-5um MSA3714-10um MSA3715-0.1um MSA3716-1um MSA3717-0.5um MSA3718-2um MSA3719-0,25um | 170-3040mm | square wave 5V TTL (RS422) | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM170/DS with MSA3712, 3714, 3716, 3717, 3718 | TGM170/DS: -no identical dimension -no periode 10um -no resolution 0.1um |
| MSA3710-1 MSA3710-2 |  |  | MSA3710-2: +/-3um,+/-5um +/-10um MSA3710-1: +/-2um,+/-3um | MSA3710-2: 20um MSA3710-1: 10um | Depending of interpolation electronics | 170-3040mm | sine: 1Vpp | 2X35mm up to1020mm 2X45mm from 1140 to 2240mm or optionally every 50mm |  | TGM170/SV with MSA3710-2 | TGM170/SI: -no identical dimension -no periode 10um |

1.3. Fagor: Iskra

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MKT |  |  | 10 um | 20um | 5um 10um | 120-1540mm | square wave 5V TTL | none | TGM130/DS |  |  |
| MT MTD MX |  |  | MT, MTD +/-10um MX: +/-5um | 20um | MT, MTD: 10um, 5um MX: 1um | MT, MTD: 120-1540mm MX: 125-1240mm | MT: square wave 5V TTL non-diff. MTD, MX: square wave 5V TTL | optionally every 50mm | TGM130/DS |  |  |
| CT CX CC, COC |  |  | CT: +/-10um CX: +/-5um CC, COC: +/-3um, +/-5um | 20um | CC, COC :Depending of interpolation electronics CT: 10um, 5um CX: 1um | 220-1240mm | CC, COC: sine current signals: 11uA, 5V CT:non-diff. square wave 5V TTL CT: square wave 5V TTL | optionally every 50mm |  | TGM170/DS for CT, CX TGM170/SI CC | TGM170: -no identical dimension |
| MVS,MOVS MVC,MOVC MVX,MOVX MVY,MOVY MVP,MOVP |  |  | +/-5um,+/-3um | 20um | MVC, MVS, MVP: :Depending of interpolation electronics MVX: 1um MVY: 0.5um | 120-1240mm | MVS: sine voltage signals 3Vpp, 5V MVS: sine voltage signals 1Vpp, 5V MVC: sine current signals MVX, MVY: square wave 5V TTL | optionally every 50mm MOVS, MOVC, MOVX, MOVY, MOVP coded RI | TGM130 for M- series TGM131 for MO- series |  |  |
| CVS, COVS CVC, COVC CVX, COVX CVP, COVP |  |  | +/-5um,+/-3um | 20um | CVC, CVS, CVP: :Depending of interpolation electronics CVX: 1um | 220-3040mm 3um 220-1240mm | CVS: sine voltage signals 3Vpp, 5V CVP: sine voltage signals 1Vpp, 5V CVC: sine current signals CVX: square wave 5V TTL | optionally every 50mm COVS, COVC, COVX, COVP coded RI |  | TGM170 for CV- series TGM173 for COV- series | TGM170, 173: -no identical dimension |

1.4. Mitutoyo: Iskra

| Product type | cross-section | dimensions | Accuracy grade (+/-um/m) | Signal periode (um) | Resolution (um) | Measuring lengths (mm) | Output signals | Reference mark | Iskra direct replacement | Iskra similar replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AT102 |  |  | +/-5um | 20um | 10um, 5um 2um, 1um | 1600 - 3000mm | square wave 5V TTL | optionally every 50mm | TGM170/DS |  |  |
| AT115 |  |  | +/-5um | 20um | 10um, 5um 2um, 1um | 100 - 1500mm | square wave 5V TTL | optionally every 50mm |  | TGM130/DS |  |
| AT112 |  |  | +/-3um | 20um | 10um, 5um 2um, 1um | 50 - 1020mm | square wave 5V TTL | optionally every 50mm |  | TGM113/DS |  |

2. Rotary encoders

2.1. Heidenhain: Iskra

| Product type | dimensions | shaft diameter | protection IP | Mecha nically permissible speed | output signals/ power supply | scanning frequency | permissible operating temperature |  | number of lines |  | Iskra direct replacement |  | Iskra similar replacement |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROD400 with synchro flange: ROD426 ROD466 ROD436 ROD456 ROD486 |  | 6 mm | IP64 | max: 12000 r/min | ROD426: 5V ROD466: 10-30V ROD436: 10-30V ROD456: sine current 11uA 5V ROD486: sine voltage 1Vpp | ROD426: 300kHz ROD466: 300kHz ROD436: 160kHz ROD456: 160kHz ROD486: 180kHz | ROD426: 100 C ROD466: 70 C ROD436: 85 C ROD456: 85 C ROD486: 100 C | ROD426: 50-10000 ROD466: 50-10000 ROD436: 50-5000 ROD456: 50-5000 ROD486: 1000-5000 |  | TGR11/DS with ROD426, ROD466, ROD436 TGR11/SI with ROD465 |  |  |  | TGR11: - no operating temperature 100 C -no version 1Vpp |  |
| ROD400 with clamping flange: ROD420 ROD430 ROD450 |  | 10 mm | IP64 | max: 12000 r/min | ROD420: 5V ROD430: 10-30V ROD450: sine current 11uA 5V | ROD420: 300kHz ROD430: 160kHz ROD450: 160kHz | ROD420: 100 C ROD430: 85 C ROD450: 85 C | ROD420: 50-5000 ROD430: 50-5000 ROD450: 50-5000 |  | TGR10/DS with ROD420, ROD430 TGR10/SI with ROD450 |  |  |  | TGR10: - no operating temperature 100 C |  |

2.2. RSF: Iskra

| Product type | dimensions | shaft diameter | protection IP | Mecha nically permissible speed | output signals/ power supply | scanning frequency | permissible operating temperature |  | number of lines |  | Iskra direct replacement |  | Iskra similar replacement |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DG118 with synchro flange: |  | 6 mm | IP64 | max: 12000 r/min | 5V 12V 24V | 300kHz | 70 C option: 100 C | 50-5400 |  | TGR11/DS with DG118 |  |  |  | TGR11: - no operating temperature 100 C |  |  |
| DG120 with clamping flange: |  | 10 mm | IP64 | max: 6000 r/min | 5V 12V 24V | 300kHz | 70 C option: 100 C | 50-5400 |  | TGR10/DS with DG118 |  |  |  | TGR11: - no operating temperature 100 C |  |  |

2.2.Fagor: Iskra

| Product type | dimensions | shaft diameter | protection IP | Mecha nically permissible speed | output signals/ power supply | scanning frequency | permissible operating temperature |  | number of lines |  | Iskra direct replacement |  | Iskra similar replacement |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type S and SW: with synchro flange: |  | 6 mm | IP64 | max: 12000 r/min | zype S: 5V 12V type SW: sine current: 11uA 5V | type S: 200kHz type SW: 100 kHz | 70 C | 50-5000 |  | TGR11/DS with type S TGR11/SI with type SW |  |  |  |  |  |  |

3. Interpolation electronics

3.1.Heidenhain: Iskra

| Product type | interpolation factor | input signals | output signals | input frequency | clock frequency | permissible operating temperature | Iskra direct replacement |  | Iskra similar replacement |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXE 602E | x5 | sine current 11uA | 5V | 25 kHz | non-clocked | 70 C | SIM110 |  |  |  |  |
| EXE 610C | x5, x10 | sine current 11uA | 5V | 25 kHz | 2MHz | 70 C | SIM110 |  |  |  |  |
| EXE 612 | x5, x10 | sine current 11uA | 5V | 100 kHz | 8MHz | 70 C | SIM110 |  |  | SIM110: +smaller dimension |  |
| EXE 650B | x25, x50 | sine current 11uA | 5V | 20 kHz | 8MHz | 70 C |  | SIM125 |  | SIM125: -no interpolation x50 |  |
| EXE 660 | x100 | sine current 11uA | 5V | 22 kHz | 10 MHz | 70 C |  | SIM128 |  | SIM128: +interpolations factor adjustable 1-128 |  |

3.2.RSF : Iskra

| Product type | interpolation factor | input signals | output signals | input frequency | clock frequency | permissible operating temperature | Iskra direct replacement |  | Iskra similar replacement |  | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZE-V5 ZE-V10 ZE-V25 ZE-V50 | ZE-V5: x5 ZE-V10: x10 ZE-V25: x25 ZE-V50: x50 | sine current 11uA | 5V | ZE-V5:100 kHz ZE-V10:50 kHz ZE-V25:20 kHz ZE-V50:10 kHz | non-clocked | 50 C | SIM110 for ZE-V5 and ZE-V10 SIM125 for ZE-V25 |  |  | no replacement for ZE-V50 |  |

4. Digital position readouts

4.1.Heidenhain: Iskra

| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ND220 ND221 ND281 ND282 ND231 ND261 ND286 | all :one axis, except ND231: two axes | one axis for length measurement, inspection equipment, sum/ difference display, angle display | all types: sine current 11uA except ND286: square wave 5V TTL | Yes | No | No | No | No | ND231: Sum/diference display, ND261: Angle display, ND281,ND286: min/max evaluation ND221,ND231: RS232C ND282,ND286:BCD output | NP10 for ND220, ND221 | NP10: -no sine current input signals -no angle measuring -no min/max measuring -no summ/difference display -no BCD output |
| ND510 ND520 | two axes | two axes version for two coordinate machine tools | sine current 11uA | Yes | 1/2 | Yes | No | No | ND510,ND520: skrinkage compensation ND520: edge finder | NP21 for ND510, ND520 | NP21: -no sine current input signals -no skrinkage compensation -no non linear compensation -no edge finder |
| ND550 ND560 | two axes | two axes version for milling machine tools | sine current 11uA | Yes | 1/2 | Yes | No | No | ND550,ND560: skrinkage compensation | NP30, NP30M: for ND550, ND560 | NP30: -no sine current input signals -no skrinkage compensation -no non linear compensation NP30M: -no skrinkage compensation |
| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| ND530 | two axes | two axes version for lathes | sine current 11uA | Yes | R/D | Yes | No | No | ND550,ND560: skrinkage compensation | NP20, NP20T: for ND530 | NP20: -no sine current input signals -no non linear compensation |
| ND920 | two axes | two axes version for two coordinate machine tools | sine current 11uA | Yes | 1/2 | Yes | No | No | +distance to go mode +shrinkage compensation +99 datum points +bolt hole +99 position steps +input/output control signals | NP20M : for ND920 | NP20M: -no 99 datum points -no non linear compensation -no 99 position steps -no input/output control signals -no shrinkage compensation |
| ND930 ND970 | two axes three axes | two axes version for lathes three axes version for lathes | sine current 11uA | Yes | R/D | Yes | No | No | +distance to go mode +shrinkage compensation +99 tools +oversize allowance +99 position steps +input/output control signals +multipass cycle +additional parallel axis (ND970) | PNC20T : for ND930 | PNC20T: -no 99 datum points -no non linear compensation -no shrinkage compensation |
| ND960 | three axes | three axes version for milling machine tools | sine current 11uA | Yes | 1/2 | Yes | No | No | +distance to go mode +shrinkage compensation +99 datum points +oversize allowance +99 position steps +input/output control signals +multipass cycle | PNC30M : for ND960 | PNC30M: (available in 1997) -no 99 datum points -no non linear compensation -no shrinkage compensation |
| VRZ739 VRZ779 | VRZ739: 2 axes VRZ779: 3 axes | two and three axes version for lathes with constant surface speed | sine current 11uA | Yes | No | Yes | No | Yes | +shrinkage compensation +spindle stop +gear change +analog output for main spindle | NP20T-V: for VRZ739 NP20T-VQ for VRZ779 | NP20T-V: -no shrinkage compensation |
| VRZ670 | three axes | three axes version for EDM machine tools | sine current 11uA | Yes | Yes | No | Yes | No | +shrinkage compensation +distance to go display +display minimum position value of electrode +compensation value input for depth +input/output control signals | NP31 | NP31 -no shrinkage value +spark erosion delay relay output |
| VRZ742C | two axis | two axes version for tool preseters | sine current 11uA | Yes | No | Yes | No | NO | +shrinkage compensation +99 tools +RS232C for tool data +control inputs | NP22 for 742 C | NP22: -no shrinkage compensation |

4.2.RSF: Iskra

| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z520 Z550 | two axes | Z520: two axes version for two coordinate machine tools | square wave 5VTTL | Yes | 1/2 R/D | Yes | No | No |  | PNC20T : for ND930 | PNC20T: -no 99 datum points -no non linear compensation -no shrinkage compensation |
| Z530 Z560 | three axes | three axes version for three coordinate machine tools (milling machine tools) | square wave 5VTTL | Yes | 1/2 R/D | Yes | No | No | +9 tools | PNC20T : for ND930 | PNC20T: -no 99 datum points -no non linear compensation -no shrinkage compensation |

4.3.Fagor: Iskra

| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V10 V11 VN100 VN101 | one axes | one axis versions for one coordinate measuring | square wave 5VTTL | Yes | No | No | No | No | VN101: angle measuring VN100/101: 1 tool VN100/101: linear compensation | NP10: : for V10, V11, V100 | NP10: -no angle measuring |
| V20 V21 VN200 VN201 VN300 VN301 | two axes | two axes versions for two coordinate machine tools | square wave 5VTTL | Yes: VN200/200/300/301 No: VN20/21(only reset, preset) | R/D 1/2 | Yes: VN200/201 VN300/301 | No | No | VN201/301: angle measuring VN200/201/300/301: 1 tool VN200/201/300/301: linear compensation | NP21: : for V20, V21, V200 NP30: for VN300 | NP21: -no angle measuring NP30: -no angle measuring |
| VN300T VN301T | two axes | two axes versions for lathes | square wave 5VTTL | Yes: | R/D | Yes: | No | No | VN300T: angle measuring VN300T/301T: 10 tools | NP20: : for VN300T | NP20: -no angle measuring -no 10 tools |
| VN300E | three axes | three axes versions for EDM machine tools | square wave 5VTTL | Yes: | 1/2 | Yes: | Yes | No | VN300E: angle measuring 1 TOOL | NP31: : for VN300e | NP31: -no 1 tool |
| VN300S | three axes | three axes versions for milling machine tools with analog spindle drive | square wave 5VTTL | Yes: | 1/2 | Yes: | No | Yes | VN300S: | No replacement |  |
| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed, S- analog | Additional options: | Iskra similar functional replacement | Remark |
| VN300TS | three axes | three axes versions for lathes with constant surface speed | square wave 5VTTL | Yes: | R/D | Yes: | No | Yes | VN300TS: | NP20T-V: : for VN300TS | NP20T-V: -no 10 tools |

4.4.Mitutoyo: Iskra

| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KS 174- 101 103 105 | one axes two axes three axes | one, two and thre axes versions for one, two (boring machine tools) and three axes (milling machine tools) measuring | square wave 5VTTL | Yes: | No | No | No | No |  | NP10 for 101 NP21 for 103 NP30 for 105 | NP10/21/30: -no remote control +PRESET +bolt hole +linear comensation |
| KM 174- 111 113 115 | one axes two axes three axes | one, two and thre axes versions for one, two (boring machine tools, lathes) and three axes (milling machine tools) measuring | square wave 5VTTL | Yes: | Yes | Yes | No | No | KM series: +macro functions +bolt hole | NP10 for 111 NP21/20 for 113 NP30 for 115 | NP10/20/21/30: -no remote control -no macro functions |
| Product type | Axes | Application | input signals | Basic functions: Reset, Preset, mm/inch,Datum points, Ref. mode, Abs/Rel, parameters entry | R/D, 1/2 | linear and non-linear machine tool compensation, tool radius compensation | EDM mode | Constant surface speed | Additional options: | Iskra similar functional replacement | Remark |
| KC 174- 121 123 125 | one axes two axes three axes | one, two and thre axes versions for one, two (boring machine tools, lathes) and three axes (milling machine tools) measuring | square wave 5VTTL | Yes: | Yes | Yes | No | No | KC series: +macro functions +bolt hole +tolerance output | NP10 for 121 NP21/20 for 123 NP30 for 125 | NP10/20/21/30: -no remote control -no macro functions -no tolerance output |
| PLE 164- 291 | three axes | three axes versions for EDM machine tools | square wave 5VTTL | Yes: | 1/2 | Yes | Yes | No |  | NP31 for PLE | NP31: -no 7 output position |
| PG-L 164- 241 243 245 | one axis two axes three axes | one, two and three axes versions for simple measuring on machine tools | square wave 5VTTL | only reset, preset | R/D | No | No | No |  | NP10/20/21/ 30: for PG-L series |  |