LINEAR ENCODERS

ISKRA

Different  types  of linear encoders by used technology:

LINEAR ENCODERS

OPTOELECTRONIC

MAGNETIC

MAGNETIC  BAND

Features:

Long length

Low accuracy

Low resolution

Low repetability

Excellent for  hard enviroment (dust, liquids, …)

Used in:

Wood &  Stone cutting machine;

NON-MAGNETIC BAND

Features:

Long length

Low accuracy

Low resolution

High repetability

Excellent for  hard enviroment (dust, liquids, …)

Used in:

Wood &  Stone cutting machine;

METAL TAPE:

Features:

Long length

Good accuracy

Low resolution

High repetability

Sensitive to dust, …

Used in:

Machine tool;

GLASS SCALE

Features:

Length to 3m

High accuracy

High resolution

High repetability

Sensitive to dust, …

Used in:

Machine tool;

Semiconductor industry

Different  types  of  optoelectronic linear encoders by application:

Optoelectronic glass linear encoders

DRO linear  encoders

For use on manualy controlled machine tool;

Retro fitting of old machine tool

NC linear encoders:

For use on NC controlled machine tool; For new builded machine, and for retrofitting .

Features:

Good accuracy;

Good interpolation error

Good reliability;

Features:

High accuracy

Low interpoaltion error

High reliability

Optoelectronic linear encoders – theory of operation (scanning):

Optoelectronic linear encoders – theory of operation (interpolation):

Optoelectronic linear encoders – functional structure (DRO encoders):

Optoelectronic linear encoders – functional structure (NC encoders):

Optoelectronic linear encoders  Reference Marks Positions:

1. Standard Reference Marks: TGM111 (every 100mm available), TGM113, 115, 130, 170 (available every 50mm)

2. Distance Coded Reference Marks: TGM131, TGM133, TGM173

- 20mm maximum traverse for re-establishing absolute position: (TGM131, TGM133,  TGM173)

- 80mm maximum traverse for re-establishing absolute position:  (TGM190)

Incremental track

20

10,02

9,98

20

10,04

9,96

20

10,06

9,94

10,08

Reference  track

Optoelectronic linear encoders – mechanical structure (DRO encoders):

Optoelectronic linear encoders – mechanical structure (NC encoders):

Optoelectronic linear encoders – features  ACCURACY:

- Accumulative accuracy

The extreme value of the postion error with reference to their mean value lie within +a um for a position within any max. 1m section of the measuring length.

If measuring length is shorter the 1m , extreme value aplies to that measuring length.

POSITION ERROR

Measuring length or 1m

+a

-a

| Type | Availble grades of a | Permissible change of postion error |
| --- | --- | --- |
| DRO | 5um; 10um | +5um/50mm |
| NC | 3um; 5um | +3um/50mm |

- Interpolation accuracy

Interpolation error

The position error within one signal period .

POSITION ERROR

Signals

+a

-a

+IE

Measuring length

-IE

| Type | Max Interpoaltion error | Average Interpoaltion error |
| --- | --- | --- |
| DRO | + 1um | + 0,5 um |
| NC | + 0,4 um | + 0,2 um |

- Return error

Return error

Difference between positioning from the difference directions.

Return  error

POSITION ERROR

Measuring length

+a

-a

Measuring length

| Type | Max Return error | Average Return error |
| --- | --- | --- |
| DRO | + 3,0 um | 1,5 um |
| NC | +1,5 um | 0,8 um |

Optoelectronic linear encoders – features  OUTPUT SIGNALS:

A

B

C

D

E

F

H

G

A/2

A/2

fi

uo

uo

uo

Output signals SV :

# 360o

| feature | Marcation | Unit | Nominal | Tolerances |
| --- | --- | --- | --- | --- |
| Incr. Signal amplitude | A, B | V | 1 | + 0,2 |
| Incr. Signal DC offset | C, D | V | 0 | +0,03 |
| Amplitude unbalance | (A+B)/(A+B) | % | 0 | + 10 |
| Phase shift | Fi | o | 90 | + 10 |
| Reference signal width | G | o | 360 | +90 |
| Reference signal ampl | E | V | 0,5 | +0,25 |
| Ref. Signal peak position | H | o | 135 | +45 |
| Ref. Sign. Secondary amplitude | F | V | -0,5 | +0,25 |

47Ohm

47ohm

22 pF

R1

C1

R1

120

Ohm

R2

R2

C1

2x220pF

LINEAR ENCODER

SUBSEQUENT ELECTRONIC

uo

Ua

R1 = 10kOhm

R2 = 10kOhm

C1 =10pF

C2 =  220pF

U0 = 2,5 + 0,5 V

Ua = (A+ - A-)* (R1/R2)

-3dB limit cca 450kHz

+

-

A+

A-

A

B

C

D

E

F

H

G

A/2

A/2

fi

0

0

0

Output signals SI :

# 360o

| feature | Marcation | Unit | Nominal | Tolerances |
| --- | --- | --- | --- | --- |
| Incr. Signal amplitude | A, B | uA | 11 | + 3 |
| Incr. Signal DC offset | C, D | uA | 0 | +0,3 |
| Amplitude unbalance | (A+B)/(A+B) | % | 0 | + 10 |
| Phase shift | Fi | o | 90 | + 10 |
| Reference signal width | G | o | 360 | +90 |
| Reference signal ampl | E | uA | 5 | +3 |
| Ref. Signal peak position | H | o | 135 | +45 |
| Ref. Sign. Secondary amplitude | F | uA | -5 | +3 |

R1

C1

R1

C1

2x220pF

LINEAR ENCODER

SUBSEQUENT ELECTRONIC

uo

Ua

R1 = 56kOhm

R2 = 56kOhm

C1 =  27pF

C2 =  220pF

U0 = 2,5 + 0,5 V

Ua = 2*Ia*R1

-3dB limit cca 60kHz

+

-

Ia-

Ia+

- Output signals DS (RS 422-A  EIA):

| HIGH IMPENDANCE STATE feature # E* # A # RI # B # A # B # RI | Unit | Nominal |
| --- | --- | --- |
| Signal levels | V | UH > 2,5V at IH = -20 mA UL < 0,5V at IL = 20 mA |
| Transition times Rise and fall time | ns | <50 |
| Maximum load current | mA | 20 |

LINEAR

ENCODER

SUBSEQUENT ELECTRONIC

Line reciever:

DS26LS32C

Z0 = 120 Ohm

R1 = 5,1kOhm

R2 = 1,8 kOhm

A

A

E

Zo

R1

R1

R2

Optoelectronic linear encoders – features  DEGREE OF PROTECTION:

- IP 53 (IEC 529)

- IP 64 (IEC 529) if  encoder is pressurized

1) Connection on linear scale body:

2) Connection on reading head:

Compressed air requirements:

| Item | Specifications |
| --- | --- |
| Pressure range | 20 to 200kPa |
| Filteration grade | Air filter 5um; Mist separator: 0,3um |
| Working fluid | Air |
| Working fluid temperature | 20oC |

Optoelectronic linear encoders – features  ELECTRICAL CONNECTION:

- Sensor lines

V

## 5V

LINEAR

ENCODER

SUBSEQUENT ELECTRONIC

Optoelectronic linear encoders – features DEFINED THERMAL BEHAVIOR:

- Reason of temperature influence on wrong measurment can be different thermal expansion between Scale Aluminimum housing (alfa = 25 ppm/k)  and base plate made from Cast Iron (alfa = 11 ppm/K)

At 20 oC

At 30 oC

Due to different thermal expansion scale's Al body expand more than Base plate. Because Scale is rigidly fixed on both ends it became bended. Bending of the scale cause Abbe error.

Base plate

Scale

Scale

Screw

Screw

Base plate

- This problem is solved by  usage of fixture mechanism which compensate different thermal expansion coeficient (availible on TGM 173 and TGM 133 whith mounting bar).

Connected to base plate

Connected to scale

Optoelectronic linear encoders – Accuracy considerations :

- Abbe error

Lscale

Lcutter

d

h

Lo

Approximation:

Lcutter - Lscale = d* 2h / Lo

Examples:

h =0,2mm;  Lo= 620mm

| d (mm) | Lcutter - Lscale (um) |
| --- | --- |
| 50 | 32 |
| 100 | 64,5 |
| 200 | 129 |
| 400 | 258 |

Examples:

h =0,05mm;  Lo= 620mm

| d (mm) | Lcutter - Lscale (um) |
| --- | --- |
| 50 | 8 |
| 100 | 16 |
| 200 | 32 |
| 400 | 64 |